EENG 284

–

Digital Design Lab

How To: Synthesize a Module

The combinedLab01 verilog file used in this example has three inputs and three outputs that are mapped to slide switches and LEDs.

Synthesizing a Verilog Module

It’s time to realize the combinedLab01 Verilog file to FPGA.  To do this follow these steps:

- In Project Navigator pane, select the File tab

- Right mouse click combinedLab01.v and select Set As Top Level Entity.

- Processing -> Start -> Start Analysis and Elaboration

- Assignments -> Pin Planner

- In the Pin Planner pop-up you should see the pin assignment pane at the bottom of the window.

- Double click in the Location cell for row c

- Scroll down the list of pins to PIN_AC9

- Complete the pin assignment for the other 5 inputs and outputs using the information contained in pin assignment table completed earlier.

- Double check your pin assignments.

- File -> Close.  Note closing your file incorporates this assignment into the project.

- Back in the Quartus window, Processing -> Start Compilation  <Ctrl-L>

- Tools -> Programmer

- In the Programmer pop-up window click Add File…

- In the Select Programming File pop-up, navigate to your project directory, then into the output files folder, the select combinedLab01.sof, click Open.  You should see something like the following.

- Connect the Altera Cyclone V GX FPGA to your computer through the USB port, connect the power supply, and push the red power-on button.  Try not to be annoyed by the infernal blinking LEDs.

- In the Programmer pop-up

  - Click Hardware Setup….

  - In the Hardware Setup select USB-Blaster [USB=0] from the Currently selected hardware pull-down

  - Click Close

- Back in the Programmer window, the box next to Hardware Setup… should reflect your choice.  Click Start,

- The Development board should stop its infernal blinking and run your program.  You may notice that the unused LEDs are dimly illuminated.

Figure 6: The Development board properly configured and running the combinedLab01 Verilog file.